50W Module Technical Datasheet

1.RF DATA

| Item |  | Spec. | Remarks |
| --- | --- | --- | --- |
| Frequency Range |  | 300-400 MHz | Bandwidth Range±10MHz |
| Frequency Range |  | 400-500 MHz | Bandwidth Range±10MHz |
| Frequency Range |  | 500-600 MHz | Bandwidth Range±10MHz |
| Frequency Range |  | 700-800 MHz | Bandwidth Range±10MHz |
| Frequency Range |  | 800-900 MHz | Bandwidth Range±10MHz |
| Frequency Range |  | 900-1000 MHz | Bandwidth Range±10MHz |
| Frequency Range |  | 1000-1100 MHz | Bandwidth Range±10MHz |
| Frequency Range |  | 6000-6150 MHz | Bandwidth Range±10MHz |
| Frequency Range |  | 6150-6300 MHz | Bandwidth Range±10MHz |
| Frequency Range |  | 6300-6450 MHz | Bandwidth Range±10MHz |
| Frequency Range |  | 6450-6600 MHz | Bandwidth Range±10MHz |
| Frequency Range |  | 6600-6750 MHz | Bandwidth Range±10MHz |
| Frequency Range |  | 6750-6900 MHz | Bandwidth Range±10MHz |
| Frequency Range |  | 6900-7050 MHz | Bandwidth Range±10MHz |
| Working Voltage工作电压 |  | 28V | 24-30V |
| Working current工作电流 |  | 3-6A | output 50W |
| Analog sweeping speed 扫描速率 |  | / | Lora |
| Output Power (Max) 输出功率 |  | 47±1 dBm | 50W |
| Gain (Max) 增益 |  | 47±1 dB |  |
| Ripple in Band 带内波动 |  | ≤ 3 dB | Peak |
| Bandwidth adjustment 带宽调节 |  | Yes | By screw |
| Center adjustment 重点频点调节 |  | Yes | By screw |
| Enable control 使能控制 |  | Customize | 24-28V |
| Input power (Max) 输入功率 |  | ≤ +10dBm | Optional (If signal source from Subscriber) |
| Input Signal Source 输入信号源 |  | Built-in high-speed noise modulation signal source | Optional (customize VCO, DDS or SDR) |
| Output VSWR 驻波 |  | ≤1.5 | Inside circulator, VSWR protect |
| High Low Temperature test | Working Temperature 工作温度 | -20～+65℃ | Working properly |
|  | Gain Stability 增益 | ±1 dB | at -20℃～+65℃ |
|  | Power stability 功率 | ±1 dB | at -20℃～+65℃ |
| Power supply Port 电源接口 |  | Positive and negative lines | 24-28V; GND; (optional: Enable control wire) |
| Shell Material 外壳材质 |  | Aluminum with natural color oxide sand blasting |  |
| Vibration Need 振动需求 |  | Meet the load need of moving vehicle |  |
| RF output connector 输出射频接口 |  | SMA-Female |  |
| LED indicator light LED指示灯 |  | Power ON/OFF |  |
| Dimension 尺寸 |  | 117*40*18/117*58*16 mm |  |
| Screw Size 螺丝尺寸 |  | M3.2 |  |
| Weight/重量 |  | 230g |  |

2. Size